Time Series Anomaly Detection for IoT Sensors

1. Problem Understanding

In modern manufacturing environments, industrial equipment is continuously monitored using IoT sensors that generate multivariate time-series data. Abnormal patterns in these sensor readings often indicate early signs of equipment malfunction, abnormal operating conditions, or upcoming failures.

Undetected anomalies can result in:

- Unplanned downtime

- Safety risks

- Increased maintenance costs

The objective of this project is to design an early warning anomaly detection system that identifies unusual behavior in sensor time-series data, enabling proactive maintenance and improved operational reliability.

2. Dataset Description and Assumptions

Dataset Choice

A synthetic multivariate time-series dataset was generated to simulate IoT sensor readings from industrial equipment.

Reason for Synthetic Data

- Real-world industrial datasets often lack labeled anomalies

- Synthetic data allows controlled injection of:

  - Sudden spikes

  - Gradual drifts

  - Abnormal variability

- Ground truth anomaly labels enable objective model evaluation

Assumptions

- Sensor data is time-ordered and continuously collected

- Normal behavior follows smooth periodic trends with noise

- Sudden spikes, gradual drifts, and variance changes represent anomalous behavior

3. Data Preparation & Exploratory Data Analysis (EDA)

Data Preparation

- Time-series data was generated with multiple sensor signals

- Missing values were checked and handled where applicable

- Data was sorted by time index to preserve temporal dependencies

Exploratory Data Analysis

EDA was conducted to:

- Understand normal sensor behavior

- Identify anomaly patterns visually

- Observe trends, seasonality, and noise levels

Key Observations

- Normal sensor readings exhibit periodic behavior with small noise

- Anomalies appear as:

  - Sharp spikes

  - Sustained deviations from normal patterns

- Visualization confirmed that injected anomalies align with real-world failure-like behavior

4. Feature Engineering Strategy

Raw sensor readings were transformed into informative features to enhance anomaly detection.

Engineered Features

- Rolling Mean: Captures local trends and smooth behavior

- Rolling Standard Deviation: Identifies abnormal variability

- Raw sensor values: Preserve original signal characteristics

Rationale

- Sudden spikes cause abrupt changes in rolling statistics

- Gradual drifts affect rolling means over time

- Variance-based anomalies are captured by rolling standard deviation

Feature Scaling

- Features were normalized using standard scaling

- Scaling ensures fair contribution of all features during model training, especially for distance-based and neural network models

5. Anomaly Detection Approaches

5.1 Approach

1: Isolation Forest

Isolation Forest is an unsupervised anomaly detection algorithm that isolates anomalies by randomly partitioning the feature space.

- Anomalies are easier to isolate due to their rarity

- Scales well to large datasets

- Suitable for real-time industrial monitoring

Hyperparameters

- Contamination: Set based on expected anomaly proportion

- Number of trees: Balanced between accuracy and computation cost

5.2 Approach

2: Autoencoder Neural Network

An autoencoder is a neural network trained to reconstruct normal data patterns. Anomalies produce higher reconstruction errors.

- Learns complex, non-linear relationships

- Effective for multivariate time-series data

- Suitable for detecting subtle and gradual anomalies

Detection Strategy

- Reconstruction error is calculated for each data point

- A threshold based on reconstruction error distribution is used to classify anomalies

6. Model Evaluation and Comparison

Evaluation Metrics

Since ground truth labels were available, models were evaluated using:

- Precision

- Recall

- F1-score

Validation Approach

- Quantitative evaluation using labeled anomalies

- Visual inspection of detected anomalies over time

- Domain reasoning to verify anomaly relevance

Comparison Summary

| Aspect | Isolation Forest | Autoencoder |
| --- | --- | --- |
| Complexity | Low | High |
| Interpretability | Moderate | Low |
| Anomaly Types | Point anomalies | Complex patterns |
| Deployment | Easy | Resource-intensive |

Key Insight

Isolation Forest is suitable for lightweight, real-time detection, while autoencoders are better at capturing complex temporal relationships.

7. Business Insights

- Early anomaly detection enables preventive maintenance

- Reduced downtime and maintenance costs

- Improved equipment reliability and safety

- Models can be integrated into monitoring dashboards for continuous alerts

8. Limitations and Future Improvements

Limitations

- Synthetic data may not capture all real-world noise patterns

- Threshold selection impacts detection accuracy

- Models assume stable operating conditions

Future Improvements

- Use real-world industrial sensor datasets

- Implement online / streaming anomaly detection

- Handle concept drift over time

- Integrate root-cause analysis for detected anomalies

9. Conclusion

This project demonstrates an end-to-end anomaly detection pipeline for IoT sensor time-series data. By combining feature engineering with both classical and deep learning approaches, the system effectively detects abnormal equipment behavior and provides actionable insights for industrial monitoring.

Top of Form

Bottom of Form